# HW Connections

- GPIO adapter DPNC422_06A is connected on GPIO J17, J18 is left unconnected. If wrong connection done, will burn the F4 fuse (3V3) and might damage the GPIO board.

- Connect FIREFLY to DPNC422_06A FRB0 slot and to FRB Power Board connector small flying board

- Connect PS cords to PS and FRB power board small flying board:

  - +/-3V4 + GND cable to PS with black wire on the middle point = GND of dual PS

  - +/-5V7 + GND cable to PS with black wire on the middle point = GND of dual PS

  - +12V + GND cable on PS

  - For the +/-3V4 and +/-5V7 cables, take care of same connector on Power board small flying board, look at labels.

  - It is not mandatory to connect all middle points of +/3V4, +/-5V7 and 12V GND together.

  - Do not connect PS earth to middle point / GND

# Power supply start

- Power on +/-5V7, +/-3V4 and 12V at the same time

- Then the GPIO 24V

Consumption:

| Voltage | +5V7 | -5V7 | +3V4 | -3V4 | +12V | +24V |
| --- | --- | --- | --- | --- | --- | --- |
| Typical Current (mA) | 187 | 30 | 497 | 12 | 4 |  |

The GPIO front end .exe files are located on http://dpnc.unige.ch/~favrey/Misc/Herd/HerdFrontEnd-FEB/

Once started, the GUI should display the version 6/0.1/8/0.

# Protocol

| 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #EVENT (6b) Range 0-63 |  |  |  |  |  | #FEB (5b) Range 0-17 (0-3 now) |  |  |  |  | #CH (9b) Range 0-383 |  |  |  |  |  |  |  |  | ADC VALUE (12b) Range 0-4095 |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  | 0 |  |  |  |  |  |  |  |  | 0x112 |  |  |  |  |  |  |  |  |  |  |  | 4 FEBs x 384 CHs = 1536 word32 per event |
| 0 |  |  |  |  |  | 0 |  |  |  |  | 1 |  |  |  |  |  |  |  |  | 0x121 |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 1 |  |  |  |  | 0 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 2 |  |  |  |  | 0 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 2 |  |  |  |  | 1 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 3 |  |  |  |  | 0 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 3 |  |  |  |  | 1 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  | 2 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 0 |  |  |  |  | 3 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 1 |  |  |  |  | 2 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 1 |  |  |  |  | 3 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 2 |  |  |  |  | 382 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 2 |  |  |  |  | 383 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 3 |  |  |  |  | 382 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  | 3 |  |  |  |  | 383 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  | 0 |  |  |  |  | 0 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  | 0 |  |  |  |  | 1 |  |  |  |  |  |  |  |  | … |  |  |  |  |  |  |  |  |  |  |  |  |